| Parameter | 620 Joints | Which Joints? | Best 100 Joints | Which Joints? | Reduction |
| --- | --- | --- | --- | --- | --- |
| Max Hi-Lo | 4.89mm | 70681_1 & 70590_2 | 1.54mm | 70141_1 & 70542_1 | ~69% |
| Max CL Eccentricity | 7.44mm | 70681_1 & 70590_2 | 2.32mm | 70667_1 & 70245_2 | ~69% |
| Max SCF | 1.36363 | 70681_1 & 70590_2 | 1.11485 | 70667_1 & 70245_2 | ~18% (~83% longer fatigue life) |
| Notes - Pipe is for a flowline project (tolerances are not as good as riser pipe). - Selection is based on the best SCF. - SCF is based on C&Z formula; however, any desired formula can be implemented (DNV, company specific, etc.). - Calculations are based the OD; however, it can also be based on the ID if desired. - All possible combinations are considered, any joint, any end to any joint, any end. - Fatigue life comparison is based on S-N curve slope of 3. - No. of iterations to calculate the results for >600 joints: ~1.5 million. - No. of iterations to select the best 100 joints: ~79 million. |  |  |  |  |  |